LAGRANGE L5 IN-SITU CONSORTIUM

MEPS Failure Mode and Effects Analysis

|  | Document ID: | LGR-IS-KIE-AN-0001_iss1_rev0_MEPS-FMEA.docx |
| --- | --- | --- |
|  | Issue: | 1 |
|  | Revision: | 0 |
|  | Date: | 07/01/2019 |

| Written | Checked | Approved configuration control | Approved QA | Approved Experiment Manager | Approved Principal Investigator |
| --- | --- | --- | --- | --- | --- |
| César Martín | Robert F. Wimmer | César Martín |  |  |  |
| Signature and date | Signature and date | Signature and date | Signature and date | Signature and date | Signature and date |

File: LGR-IS-KIE-AN-0001_iss1_rev0_MEPS-FMEA.docx

DISTRIBUTION LIST

The following list indicates the individuals and agencies in receipt of review copies of the present document:

| Agency / Organization | Name & Title | Contact information |
| --- | --- | --- |
| CAU | MEPS Kiel team | Bjoern Schuster; schuster@physik.uni-kiel.de. Cesar Martin; martin@physik.uni-kiel.de. Lars Seimetz; seimetz@physik.uni-kiel.de. Shrinivasrao Kulkarni; kulkarni@physik.uni-kiel.de. Stephan Boettcher; boettcher@physik.uni-kiel.de. Wimmer-Schweingruber; wimmer@physik.uni-kiel.de. Christoph Terasa; terasa@physik.uni-kiel.de. Ali Ravanbakhsh; ravanbakhsh@physik.uni-kiel.de. Mahesh Yedla; yedla@physik.uni-kiel.de. Robert Elftmann; elftmann@physik.uni-kiel.de. Stefan Kolbe; kolbe@physik.uni-kiel.de. Violetta Knierim; knierim@physik.uni-kiel.de. Mike Richards; mlr@richards-consulting.eu. |
| UCL | Project Office | Richard Cole; r.cole@ucl.ac.uk. Jonathan Rae; jonathan.rae@ucl.ac.uk. Hubert Hu; d.hu@ucl.ac.uk. Dhiren Kataria; d.kataria@ucl.ac.uk. Bob Bentley; b.bentley@ucl.ac.uk. |

CHANGE RECORD

| Issue | Revision | Date | Modified by | Section / Paragraph modified | Change implemented |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 07/01/2019 | C. Martin | All | First draft |

TABLE OF CONTENTS

1	INTRODUCTION	6

1.1	Purpose	6

1.2	Scope	6

1.3	Project outline	6

1.4	Details of MEPS	9

1.5	MEPS / CAU-Kiel	10

2	GLOSARY AND DEFINITIONS	11

2.1	Acronyms and Abbreviations	11

3	APPLICABLE AND REFERENCE DOCUMENTS	12

3.1	Applicable Documents (from Solar Orbiter Mission)	12

3.2	Normative documents	13

3.3	Reference documents	13

4	Reliability analysis	14

4.1	Analysis methods	14

4.1.1	FMEA	14

4.1.2	FTA	14

4.1.3	HSIA	14

4.2	Analysis assumptions and definitions	14

4.3	Severity levels and failure effects	15

4.4	Break-down to board and component group levels	15

4.5	Functional level breakdown	19

4.6	Functional FMEA worksheets	20

LIST OF FIGURES

Figure 1: MEPS single unit CAD-study with two sensor heads, each of which has two double-ended telescopes. One MEPS unit provides eight view cones in total, four for electrons and four for ions.	7

Figure 2: Working principle of an EPE double-ended telescope. The polyimide layer stops ions but lets electrons pass, whereas on the other side electrons are deflected by magnets and ions pass unaffected. The thin detector on the right side enables the separation of different ion species via the d E/dx vs. E-method (in the MeV/nuc energy range; for the separation at lower energies see text).	8

Figure 3: Electron Proton Telescope on Solar Orbiter (EPT) EM measurements with a Bi- 207 source. The conversion electron lines at 482 keV and 554 keV are seen on the PI-side (Det 1) but vanish on the magnet-side (Det 2).	9

Figure 4 MEPS as part of the Lagrange L5 In-situ Consortium	10

Figure 5  MEPS unit breakdown to board and sensor level	16

Figure 6   MEPS-A and MEPS-B and HET preamp boards (from Solar Orbiter EPT-HET heritage)	17

Figure 7  MEPS Analog ADC Shaper board (from Solar Orbiter EPT-HET heritage)	17

Figure 8  MEPS Digital board (from Solar Orbiter EPT-HET heritage)	18

Figure 9  MEPS Low Voltage Power Supply (from Solar Orbiter EPT-HET heritage)	18

Figure 10  MEPS block diagram	19

LIST OF TABLES

Table 1: Proposed MEPS instrument heritage of MEPS from EPT Solar Orbiter Technical Parameters	7

Table 2 MEPS unit breakdown	11

Table 3  Applicable documents	13

Table 4   Normative documents	13

Table 5   Reference documents	14

Table 6   Severity classification levels	15

INTRODUCTION

Purpose

This document provides FMEA analysis in support of reliability analysis for MEPS sensors of the EPD instrument of Solar Orbiter.

Scope

This document applies to the current design of the electronics for MEPS. As of this version, the basic design is (AD-09).

Project outline

The MEPS instrument is a particle instrument measuring the energy spectra and angular distributions of energetic electrons (30-600 keV) and ions (30-6000 keV/nuc). The full experiment consists of one units which provides one double-ended telescope pair (sensor heads) with four view cones per pair, two dedicated for electrons and two for ions. Utilizing any spin of the spacecraft, MEPS observations cover the full sky with a total of eight electron and eight ion view cones. As the L5 S/C is not a spinner, MEPS still provides crucial pitch-angle information to determine whether the event is scatter free. This is an important indicator for interpreting the timing information of the space weather event. Figure 1 shows a single MEPS unit, with eight view cones.

MEPS-B

Sensor

MEPS-A

Sensor

Electronics

Module

Figure 1: MEPS single unit CAD-study with two sensor heads, each of which has two double-ended telescopes. One MEPS unit provides eight view cones in total, four for electrons and four for ions.

Units: 1

Sensors: 2 per unit

FoVs: 4 double FoVs per unit. The FOVs for each sensor are separated by 70°

Angular resolution: 45° full angle

Energy range:

- 30keV/n-6MeV/n for the 20-300-300 um detector stack solution

Volume: 300x160x175 mm3 (including potential radiators and MLI)

Power: 5W for 1 unit

Mass: 2.4 kg for 1 unit

Data Products: Particle VDF

Cadence: 5(1) minutes

TRL: 6 (most parts 8-9)

Table 1: Proposed MEPS instrument heritage of MEPS from EPT Solar Orbiter Technical Parameters

The two ends of a telescope observe electrons and ions, respectively. Each telescope consists of a stack of three solid-state detectors (Figure 2). On one end, the outer detector is covered with a thin polyimide layer or parylene foil (TBC), stopping ions below a few hundred keV/nuc but letting electrons pass almost unaffected. This uppermost detector (300 μm thick Si) on this side is operated in anticoincidence with the second (middle) detector and thus observes the energy spectrum of stopping electrons. The other end of the telescope has a broom magnet (instead of a PI layer) that defects electrons below a few hundred keV (see Figure 3 for electron deflection measured with the Solar Orbiter EPT EM). This side of the telescope employs a 20 μm thick Si detector in front of the 300 μm Si detector in the middle, which thus form an ion telescope observing at energies from 30-6000 keV/nuc. Ions passing the thin first detector can be identified at high energies (MeV/nuc range) using the d E/dx vs. E technique, operable at Ekin ≥ 1 MeV/nuc for protons and alphas and ≥ 2 MeV/nuc for heavier species. Lower ion energies can nevertheless be resolved using the middle detector as an anticoincidence: Anything above 1.2 MeV energy deposit in the thin detector is heavier than protons, which in return means that the thin detector may be used as a Z ≥ 2 channel SSDs for deposited energies > 1.2 MeV (or 300 keV/nuc for He). The same logic applies for penetrating He, enabling a Z ≥ 6 channel for energies > 4.8 MeV; or 420 keV/nuc for C, 360 keV/nuc for N and 310 keV/nuc for O.

Figure 2: Working principle of an EPE double-ended telescope. The polyimide layer stops ions but lets electrons pass, whereas on the other side electrons are deflected by magnets and ions pass unaffected. The thin detector on the right side enables the separation of different ion species via the d E/dx vs. E-method (in the MeV/nuc energy range; for the separation at lower energies see text).

The permanent magnets of one sensor head (double-ended telescope pair) form a quadrupole, significantly reducing the far field. The far field is reduced even further by the other sensor head on the same unit, since its quadrupole forms an angle of 70° with respect to the first sensor head. The MEPS unit integrates the sensors and electronics in a single package. Right below the sensors in the electronic box are the preamp boards, followed by an analogue board and the back-end electronics (digital and LVPS-board[s]). Pre-flight ground energy calibration (1 % level) of all detector elements, on-axis active area calibration, selected off-axis directions active area calibration and dead-time calibration will be performed. Calibration quality will be monitored in-flight using measured data and cross-calibration with suprathermal electron/ion instruments.

Figure 3: Electron Proton Telescope on Solar Orbiter (EPT) EM measurements with a Bi- 207 source. The conversion electron lines at 482 keV and 554 keV are seen on the PI-side (Det 1) but vanish on the magnet-side (Det 2).

Details of MEPS

Figure 4 shows the LGR In-situ suite star configuration.

Figure 4 MEPS as part of the Lagrange L5 In-situ Consortium

MEPS / CAU-Kiel

The MEPS unit has heritage from the STEREO/SEPT and Solar Orbiter/EPT instruments. The combination of the two sensors sharing a common electronics box  is called the MEPS unit.

The breakdown of an MEPS unit into unit blocks and subsystems is depicted in the product tree (AD-04), and the mechanical assembly in the MICD (AD-05). Details of the electronics board and sensor level assemblies are in the AIT plan (AD-06).

For purposes of the FMEA, the board level assemblies listed in Table , including associated electronics, will be considered. Circuit diagrams are in AD-07.

| Ref | No. | Unit block | No. | Unit subsystem | No. | Unit board level assemblies |
| --- | --- | --- | --- | --- | --- | --- |
| 1.0.0 | 1 | MEPS-A sensor |  |  |  |  |
| 1.0.1 |  |  | 1 | Foil |  |  |
| 1.0.2 |  |  | 4 | SSD |  |  |
| 2.0.0 | 1 | MEPS-B sensor |  |  |  |  |
| 2.0.1 |  |  | 1 | Foil |  |  |
| 2.0.2 |  |  | 2 | SSDs |  |  |
| 3.0.0 | 1 | E-box |  |  |  |  |
| 3.1.0 |  |  | 1 | MEPS-A Preamp board |  |  |
| 3.1.1 |  |  |  |  | 8 | Preamp |
| 3.1.2 |  |  |  |  | 2 | LDO |
| 3.2.0 |  |  | 1 | MEPS-B Preamp board |  |  |
| 3.2.1 |  |  |  |  | 22 | Preamp |
| 3.2.2 |  |  |  |  | 2 | LDO |
| 3.3.0 |  |  | 1 | MEPS Analog board |  |  |
| 3.3.1 |  |  |  |  | 1 | FPGA |
| 3.3.2 |  |  |  |  | 30 | Shapers |
| 3.3.3 |  |  |  |  | 30 | ADC |
| 3.3.4 |  |  |  |  | 1 | HK ADC |
| 3.4.0 |  |  | 1 | MEPS Digital board |  |  |
| 3.4.1 |  |  |  |  | 1 | FPGA |
| 3.4.2 |  |  |  |  | 2 | Memory (EEPROM & SRAM) |
| 3.4.3 |  |  |  |  | 2 | LVDS |
| 3.4.4 |  |  |  |  | 2 | LDO for Shapers |
| 3.4.5 |  |  |  |  | 1 | Clock |
| 3.5.0 |  |  | 1 | MEPS LVPS board |  |  |
| 3.5.1 |  |  |  |  | 1 | Input EMI |
| 3.5.2 |  |  |  |  | 1 | Flyback Converter |
| 3.5.3 |  |  |  |  | 2 | Transformers |
| 3.5.4 |  |  |  |  | 4 | ISR for digital supply |
| 3.5.5 |  |  |  |  | 1 | ISR for analog supply |
| 3.5.6 |  |  |  |  | 2 | LDO +-6V analog |
| 3.5.7 |  |  |  |  | 1 | Bias LRCS |

Table 2 MEPS unit breakdown

GLOSARY AND DEFINITIONS

Acronyms and Abbreviations

| CME | Coronal Mass Ejection |
| --- | --- |
| DOCS | Deep-space Optical Communication System |
| DPU | Data Processing Unit |
| EDAC | Error Detection and Correction |
| EEE | Electrical, Electronic and Electro-mechanical |
| EMC | Electro-Magnetic Cleanliness |
| EOL | End of Life |
| EPT | Electron Proton Telescope |
| HV | High Voltage |
| IMF | Interplanetary Magnetic Field |
| LCL | Latch Current Limiter |
| LET | Linear Energy Transfer |
| LGR | Lagrange |
| MAG | Magnetometer |
| MCP | Microchannel Plate |
| MEPS | Medium Energy Particle Spectrometer |
| MICD | Mechanical Interface Drawing |
| MRAM | Magneto-Resistive Memory |
| NGRM | Next Generation Radiation Monitor |
| PLA | Plasma Analyser |
| RADEM | Radiation Monitor |
| RAM | Random Access Memory |
| RAMS | Reliability, Availability, Maintainability, Safety |
| RM | Radiation Monitor |
| SEE | Single Event Effects |
| SEP | Solar Energetic Particles |
| SIR | Stream Interaction Region |
| SIXS | Solar Intensity X-ray and particle Spectrometer |
| SO | Solar Orbiter |
| SRAM | Static Random Access Memory |
| SSA | Space Situational Awareness |
| IS | Solar Wind Analyser |
| TC | Telecommand |
| TID | Total Ionizing Dose |
| TM | Telemetry |
| TOF | Time of Flight |
| XFM | X-ray Flux Monitor |

APPLICABLE AND REFERENCE DOCUMENTS

Applicable Documents (from Solar Orbiter Mission)

| ID. | Title | Reference | Iss./Rev. | Date |
| --- | --- | --- | --- | --- |
| AD-01 | Product Assurance Requirements for instruments | SOL-EST-RS-1937 | Draft 2 | 2011-02-25 |
| AD-02 | EPD EID-B | SO-EPD-IF-0001 | 3 / 0 | 2012-10-12 |
| AD-03 | EPD DRD SO | SO-EPD-EID-00001 | 2 / 4 draft | 2011-02-17 |
| AD-04 | Kiel Product Tree | SO-EPD-KIE-PT-0001 | 1 / 0 | 2011-08-31 |
| AD-05 | EPT-HET MICD | SO-EPD-KIE-DR-0001 | 3 / 0 | 2011-02-17 |
| AD-06 | EPT-HET and STEP AIT plan | SO-EPD-KIE-PL-0010 | 2 / 1 | 2013-10-30 |
| AD-07 | EPT-HET and STEP Circuit Diagrams | SO-EPD-KIE-CD-0010 | 2 / 0 | 2013-10-08 |
| AD-08 | EPT-HET & STEP Block Diagrams | SO-EPD-KIE-BD-0010 | 2 / 0 | 2013-10-08 |
| AD-09 | EPT-HET & STEP EEE list | SO-EPD-KIE-LI-0050 | 2 / 0 | 2013-09-24 |

Table 3  Applicable documents

Normative documents

| ID. | Title | Reference | Iss. / Rev. | Date |
| --- | --- | --- | --- | --- |
| ND-01 | Space product assurance: Dependability | ECSS-Q-ST-30C | C | 2009-03-06 |
| ND-02 | Space product assurance: Failure modes, effects (and criticality) analysis (FMEA/FMECA) | ECSS-Q-ST-30-02C | C | 2009-03-06 |
| ND-03 | Jet Propulsion Laboratory Reliability Analyses Handbook | JPL D-5703 |  | 1990-07 |
| ND-04 | Space product assurance: Fault tree analysis - Adoption notice ECSS/IEC 61025 | ECSS-Q-ST-40-12C | C | 2008-07-31 |
| ND-05 | Fault Tree Handbook, U.S. Nuclear Regulatory Commission | NUREG 0492 |  | 1981-01 |
| ND-06 | Fault tree handbook with aerospace applications | NASA Document | 1.1 | 2002-08 |
| ND-07 | Critical-item control | ECSS-Q-ST-10-04C | C | 2008-07-31 |
| ND-08 | Risk management | ECSS-M-ST-80C | C | 2008-07-31 |

Table 4   Normative documents

Reference documents

| ID. | Title | Reference | Iss. / Rev. | Date |
| --- | --- | --- | --- | --- |
| RD-01 | Reliability Assessment Report for MEPS | LGR-IS-KIE-RP-0004 | 1 / 0 | 29/12/2018 |

Table 5   Reference documents

Reliability analysis

ESA dependability document ND-01 calls for supporting analyses for reliability assessment (RD-01). EPD DRD SO (AD-03) calls for FMEA (p. 54) and FTA (p.47), and in addition the deliverable documentation to ESA includes HSIA, as part of the FMEA.

This document is the response to these requirements.

Analysis methods

FMEA

The primary ESA document for FMEA method is ND-02. Additional guidance comes from the NASA handbook ND-03.

FTA

The primary ESA document for FTA method is ND-04, which is an adoption notice for IEC 61025, and recommends the use of ND-05.

Additional guidance comes from the NASA handbook ND-03, and the NASA document ND-06 is an update of the recommended ND-05.

For MEPS, FTA has not been performed.

HSIA

The primary ESA document for HSIA method is ND-02 §7. Additional guidance comes from ND-03.

For MEPS, HSIA has not been performed.

Analysis assumptions and definitions

The following lists of parts are for flight model design and demonstration model (DM) design and manufacture. Where a part or unit is DM it will be so indicated.

Severity levels and failure effects

General requirements for FMEA include identification of critical items (ND-02 §4.1.c), and criteria for identifying critical items are listed in §4.3.

Severity categories for failure modes are defined in ND-02 §4.2, however, for consistency with the classification in the Critical Items List (AD-08), the system in the CIL (Severity Level SN) will be followed. The suffix ‘R’ indicates redundancy, and ‘SP’ single point failure.

For failures at subsystem, assembly or equipment level, the term "mission" is understood as meeting sensor specification requirements.

| Severity number SN | Description | FMEA severity level (1) | Dependability effects | Safety effects |
| --- | --- | --- | --- | --- |
| 5 | Catastrophic | 1 | Failure propagation | Loss of system |
| 4 | Critical | 2 | Loss of mission | Major damage to interfacing flight systems |
| 3 | Major | 3 | Major mission degradation |  |
| 2 | Minor | 4 | Minor mission degradation |  |
| 1 | Negligible | 4 | Other failure/ no loss of science |  |

Table 6   Severity classification levels

(1) ND-02 §4.2, but not used in this document, see AD-01

Hardware failure modes are given in ND-02 §G.

Functional failure modes include

failure to perform at all,

failure to perform completely,

perform incorrectly,

does not fail safe.

Where appropriate a mixture of functional FMEA and hardware FMEA will be used.

Break-down to board and component group levels

The analysis will be performed from component level upwards. In keeping with the breakdown in Figure 5. The FMEA work sheets are organized beginning with component groups on unit boards. The circuit diagram reference for the component groups is AD-07, and functional block diagrams are in AD-08.

The highest relevant level to consider in the break down is the MEPS unit level. The electronics for the MEPS unit comprises a connected set of boards (Figure 5), and each board in turn comprises an interconnected set of component groups.

Figure 5  MEPS unit breakdown to board and sensor level

Figure 6   MEPS-A and MEPS-B and HET preamp boards (from Solar Orbiter EPT-HET heritage)

Figure 7  MEPS Analog ADC Shaper board (from Solar Orbiter EPT-HET heritage)

Figure 8  MEPS Digital board (from Solar Orbiter EPT-HET heritage)

Figure 9  MEPS Low Voltage Power Supply (from Solar Orbiter EPT-HET heritage)

Functional level breakdown

Reference for the functional level breakdown of the MEPS sensor is the MEPS Block Diagram, 2013-01-23, found in AD-08. The following tables use the blocks and terminology of that block diagram, and there is no one-to-one correspondence to physical electronics boards.

Figure 10  MEPS block diagram

Functional FMEA worksheets

| 3.1.0 MEPS-A Preamps |  | Function | Failure Mode | Causal Factors | Local effects | System effects | FMEA Severity level | Detection method | Current controls | Recommendations |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Id no | Item / type |  |  |  |  |  |  |  |  |  |
| 3.1.1 | PA / Charge Sensitive Amplifier | Amplify analog signals from detector | Fails at input | any | Loss of signal channel | Science degraded | 3R |  |  |  |
|  |  |  | Op Amp fails | any | Loss of signal channel | Science degraded | 3R |  |  |  |
|  |  |  | Amplification changes | any | Degraded signal | Science degraded | 3R |  |  |  |
| 3.1.2 | LDO for Preamp | Regulate from 6V to 5V | Fails OC | any | Loss of output voltage | Loss of EPT | 3R |  |  |  |
|  |  |  | Fails SC, 6V output | any | Item tolerant to 6V | Loss of EPT | 3R |  |  |  |
| 3.1.2 | LDO for reference Vfet | Regulate from 6V to 5V and Vref | Fails OC | any | Loss of output voltage | Loss of EPT | 3R |  |  |  |
|  |  |  | Fails SC, 6V output | any | Item tolerant to 6V | Loss of EPT | 3R |  |  |  |

| 3.2.0 MEPS-B Preamps |  | Function | Failure Mode | Causal Factors | Local effects | System effects | FMEA Severity level | Detection method | Current controls | Recommendations |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Id no | Item / type |  |  |  |  |  |  |  |  |  |
| 3.2.1 | Preamp / Charge Sensitive Amplifier | Amplify analog signals from detector | Fails at input | any | Loss of signal channel | Science degraded | 3R |  |  |  |
|  |  |  | Op Amp fails | any | Loss of signal channel | Science degraded | 3R |  |  |  |
|  |  |  | Amplification changes | any | Degraded signal | Science degraded | 3R |  |  |  |
| 3.2.2 | LDO for Preamp | Regulate from 6V to 5V | Fails OC | any | Loss of output voltage | Loss of HET | 3R |  |  |  |
|  |  |  | Fails SC, 6V output | any | Loss of Preamp | Loss of HET | 3R |  |  |  |
| 3.2.2 | LDO for reference Vfet | Regulate from 6V to 5V and Vref | Fails OC | any | Loss of output voltage | Loss of HET | 3R |  |  |  |
|  |  |  | Fails SC, 6V output | any | Loss of Preamp | Loss of HET | 3R |  |  |  |

| 3.3.0 MEPS Analog |  | Function |  |  | Failure Mode |  |  | Causal Factors |  |  | Local effects |  |  | System effects |  |  | FMEA Severity level |  | Detection method |  |  | Current controls |  |  |  | Recommendations |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Id no | Item / type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.3.1 | FPGA 2M gates | Command and data handling for Shaper and ADC i/f to Preamps and sensors |  |  | Loss of input channel |  |  | any |  |  | Loss of signal |  |  | Loss of science |  |  | 3R |  |  |  |  |  |  |  |  | SP |  |  |
|  |  |  |  |  | Loss of regulated 3.3VD |  |  | any |  |  | Loss of FPGA output |  |  | Loss of one EPT-HET |  |  | 3R |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Loss of configuration I/F |  |  | any |  |  | Loss of ASIC control |  |  | Loss of one EPT-HET |  |  | 3R |  |  |  |  |  |  |  |  |  |  |  |
| 3.3.2 | SH high gain | Pulse shaper |  | Op Amp fails |  |  | any |  |  | Loss of signal channel |  |  | Science is degraded |  |  | 3R |  |  |  |  |  |  |  |  |  |  |  |
| 3.3.2 | SH low gain | Pulse shaper |  | Op Amp fails |  |  | any |  |  | Loss of signal channel |  |  | Science is degraded |  |  | 3R |  |  |  |  |  |  |  |  |  |  |  |
| 3.3.3 | ADC | Analog shaper signal to FPGA | ADC fails at input |  |  | any |  |  | Loss of signal channel |  |  | Science is degraded |  |  | 3R |  |  |  |  |  |  |  |  |  |  |  |
| 3.3.4 | HK-ADC | Analog HK to digital input to FPGA | ADC fails at input |  |  | any |  |  | Loss of HK-ADC |  |  | Loss of HK |  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |

| 3.4.0 MEPS Digital |  | Function | Failure Mode | Causal Factors |  |  | Local effects |  | System effects |  | FMEA Severity level |  | Detection method | Current controls |  |  | Recommendations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Id no | Item / type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.4.1 | FPGA 2M gates | I/F to analog and ADC Shaper electronics | Loss of input channel | any |  |  | Loss of signal |  | Loss of science |  | 3R |  |  |  |  |  | SP, CIL |  |
|  |  |  | Loss of I/F to SRAM | any |  |  | Loss of memory |  | Loss of one EPT-HET |  | 3R |  |  |  |  |  |  |  |
|  |  | I/F to ICU | Loss of comm I/F | any |  |  | No comm with ICU |  | Loss of one EPT-HET |  | 3R |  |  |  |  |  |  |  |
| 3.4.2 | SRAM | Memory 2Mx8 | Any output OC | any |  |  | Loss of data |  | Loss of FPGA function |  | 3R |  |  |  |  |  |  |  |
| 3.4.2 | EEPROM | Memory 1Megabit | Any output OC | any |  |  | Loss of data |  | Loss of FPGA function |  | 3R |  |  |  |  |  |  |  |
| 3.4.3 | LVDS I/F | Command, clock, data I/F to ICU | Driver fails in OC | any |  |  | Loss of TX with ICU |  | Loss of one EPT-HET |  | 3R |  |  |  |  |  |  |  |
|  |  |  | Receiver fails in OC | any |  |  | Loss of RX with ICU |  | Loss of one EPT-HET |  | 3R |  |  |  |  |  |  |  |
| 3.4.4 | LDO | +6VA regulated supply for Shaper | Fails OC |  | any | Loss of output voltage |  | Loss of Shapers |  | 3R |  |  |  |  |  |  |  |
|  |  |  | Fails SC, 6V output |  | any | Item tolerant to 6V |  | Loss of Shapers |  | 3R |  |  |  |  |  |  |  |
| 3.4.4 | LDO | -6VA regulated supply for Shaper | Fails OC |  | any | Loss of output voltage |  | Loss of Shapers |  | 3R |  |  |  |  |  |  |  |
| 3.4.5 | Clock | Provides 48MHz | Fail in OC | any |  |  | FPGA fails |  | Loss of one EPT-HET |  | 3R |  |  |  |  |  |  |  |

Loss of LVPS leads to loss of sensor, but does not propagate to ICU.

| 3.5.0 MEPS LVPS |  | Function | Failure Mode | Causal Factors | Local effects | System effects | Severity class / prob. | Detection method | Current controls | Recommendations |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Id no | Item / type |  |  |  |  |  |  |  |  |  |
| 3.5.1 | EMI | Input filter | Fails in OC | any | No power to LVPS | Loss of one EPT-HET | 4R |  |  |  |
| 3.5.2 | PWM | Controller for power conversion | Vref fails | any | Output disabled | Loss of prim. side voltage | 4R |  |  |  |
| 3.5.2 | CLK | 3.2MHz clock for controller sync | Fail in OC | any | None | Loss of sync | 4R |  |  |  |
| 3.5.3 | Power transformer | Isolation SC power from MEPS power | Fails in OC on primary side | any | Loss of secondary side | Loss of one EPT-HET | 4R |  |  |  |
|  |  |  | Fails in OC on secondary side | any | Loss of +6VA or +6VD or BIAS or 12VSEC | Loss of one EPT-HET | 4R |  |  |  |
| 3.5.4 | Switching regulator to FPGA on Digital | Supply +1.5VD | Fail in OC | any | Loss of voltage to FPGA | Loss of one EPT-HET | 4R |  |  |  |
| 3.5.4 | Switching regulator to FPGA on Analog board | Supply +1.5VD | Fail in OC | any | Loss of voltage to Analog board FPGA | Loss of one EPT-HET | 4R |  |  |  |
| 3.5.4 | Switching regulator to SRAM | Supply +1.8VD | Fail in OC | any | Loss of voltage to SRAM | Loss of one EPT-HET | 4R |  |  |  |
| 3.5.4 | Switching regulator to FPGA, SRAM, EEPROM, and CLK | Supply +3.3VD | Fail in OC | any | Loss of voltage to FPGA | Loss of one EPT-HET | 4R |  |  |  |
| 3.5.5 | Switching regulator to Analog board | Supply +3.3VA | Fail in OC | any | Loss of voltage to Analog board FPGA | Loss of one EPT-HET | 4R |  |  |  |
| 3.5.6 | LDO | +6VA regulated supply for Analog | Fails OC | Any | Loss of output voltage |  | 4R |  |  |  |
|  |  |  | Fails SC, 6V output | any | Item tolerant to 6V | none | 4R |  |  |  |
| 3.5.6 | LDO | -6VA regulated supply for Analog | Fails OC | any | Loss of output voltage |  | 4R |  |  |  |
| 3.5.7 | LRCS | Low ramp bias supply | Fails in OC | any | Loss of bias voltage | No bias voltage at sensor | 4R |  |  |  |