LAGRANGE L5 IN-SITU CONSORTIUM

MEPS Part Stress Analysis Report

|  | Document ID: | LGR-IS-KIE-RP-0003_iss1_rev0_MEPS_Parts-Stress-Analysis-report.docx |
| --- | --- | --- |
|  | Issue: | 1 |
|  | Revision: | 0 |
|  | Date: | 07/01/2019 |

| Written | Checked | Approved configuration control | Approved QA | Approved Experiment Manager | Approved Principal Investigator |
| --- | --- | --- | --- | --- | --- |
| C. Martin | C. Martin | C. Martin |  |  |  |
| Signature and date | Signature and date | Signature and date | Signature and date | Signature and date | Signature and date |

File: LGR-IS-KIE-RP-0003_iss1_rev0_MEPS_Parts-Stress-Analysis-report.docx

DISTRIBUTION LIST

The following list indicates the individuals and agencies in receipt of review copies of the present document:

| Agency / Organization | Name & Title | Contact information |
| --- | --- | --- |
| CAU | MEPS Kiel team | Bjoern Schuster; schuster@physik.uni-kiel.de. Cesar Martin; martin@physik.uni-kiel.de. Lars Seimetz; seimetz@physik.uni-kiel.de. Shrinivasrao Kulkarni; kulkarni@physik.uni-kiel.de. Stephan Boettcher; boettcher@physik.uni-kiel.de. Wimmer-Schweingruber; wimmer@physik.uni-kiel.de. Christoph Terasa; terasa@physik.uni-kiel.de. Ali Ravanbakhsh; ravanbakhsh@physik.uni-kiel.de. Mahesh Yedla; yedla@physik.uni-kiel.de. Robert Elftmann; elftmann@physik.uni-kiel.de. Stefan Kolbe; kolbe@physik.uni-kiel.de. Violetta Knierim; knierim@physik.uni-kiel.de. Mike Richards; mlr@richards-consulting.eu. |
| UCL | Project Office | Richard Cole; r.cole@ucl.ac.uk. Jonathan Rae; jonathan.rae@ucl.ac.uk. Hubert Hu; d.hu@ucl.ac.uk. Dhiren Kataria; d.kataria@ucl.ac.uk. Bob Bentley; b.bentley@ucl.ac.uk. |

CHANGE RECORD

| Issue | Revision | Date | Modified by | Section / Paragraph modified | Change implemented |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 07/01/2019 | C. Martin | All | First issue |

TABLE OF CONTENTS

Contents

1	Introduction	6

1.1	Purpose	6

1.2	Scope	6

1.3	Project outline	6

1.4	Details of MEPS	9

1.5	Unit level details of MEPS	10

2	GLOSARY AND DEFINITIONS	11

2.1	Acronyms and Abbreviations	11

3	APPLICABLE AND REFERENCE DOCUMENTS (from Solar Orbiter)	12

4	PSA requirements	13

4.1	Applicable Dependability Requirements	13

5	Circuit breakdown	14

5.1	MEPS	14

5.2	SPICE models	17

6	Part Stress	19

6.1	EMI filter	19

6.2	Inrush circuit	19

6.3	5V Regulator	21

6.4	Switching regulator: ISR FPGA 1.5V	21

6.5	Switching regulator: ISR SRAM 1.8V	22

6.6	Switching regulator: ISR DIG 3.3V	23

6.7	Switching regulator: ISR ANA ADC 3.3V	24

6.8	Positive and Negative LDOs	25

6.9	BIAS Current Source and Monitor	26

6.10	3-Stage Voltage Multiplier	28

6.11	Planar Transformer	28

6.12	Snubber Circuit	29

6.13	PWM Modulator, Crystal Oscillator and Divider (Flyback Converter)	30

6.14	RF Transformer	31

6.15	PWM Op Heater	31

6.16	Summary	32

7	Derating analysis	33

LIST OF FIGURES

Figure 1: MEPS single unit CAD-study with two sensor heads, each of which has two double-ended telescopes. One MEPS unit provides eight view cones in total, four for electrons and four for ions.	7

Figure 2: Working principle of an EPE double-ended telescope. The polyimide layer stops ions but lets electrons pass, whereas on the other side electrons are deflected by magnets and ions pass unaffected. The thin detector on the right side enables the separation of different ion species via the d E/dx vs. E-method (in the MeV/nuc energy range; for the separation at lower energies see text).	8

Figure 3: Electron Proton Telescope on Solar Orbiter (EPT) EM measurements with a Bi- 207 source. The conversion electron lines at 482 keV and 554 keV are seen on the PI-side (Det 1) but vanish on the magnet-side (Det 2).	9

Figure 4 MEPS as part of the Lagrange L5 In-situ Consortium	10

Figure 5 EPT-HET block diagram	11

Figure 6 MEPS circuit block diagram	14

Figure 7 MEPS LVPS board circuit breakdown	15

Figure 8 MEPS Digital board circuit breakdown	15

Figure 9 MEPS Analog board circuit breakdown	16

Figure 10 circuit breakdown MEPS-A Preamp and MEPS-B Preamp board	16

LIST OF TABLES

Table 1: Proposed MEPS instrument heritage of MEPS from EPT Solar Orbiter Technical Parameters	8

Table 2 Applicable documents	12

Table 3 Normative documents	13

Table 4 Reference documents	13

Table 5 Ceramic capacitors Group Code 01-01	32

Table 6 Solid Tantalum capacitors Group Code 01-04	33

Table 7 Chip resistors (Thick & Thin film) Group Code 10-08	33

Table 8 Voltage reference / Zener diode Group Code 04-04	33

Table 9 RF/Microwave Schottky Si diodes Group Code 04-05	34

Table 10 Inductor cores Group Code 07-02	34

Table 11 Bipolar PNP Transistor Group Code 12-01	35

Table 12 JFET N-Channel Group Code 12-15	35

Table 13 FET P-Channel Group Code 12-06	35

Table 14 FET N-Channel Group Code 12-06	36

Table 15 FET N-Channel Group Code 12-06	36

Table 16 Bipolar NPN Transistor Group Code 12-01	36

Table 17 UHF NPN Transistor Group Code 12-01	37

Table 18 Bipolar NPN Transistor Group Code 12-01	37

Table 19 Linear Operational Amplifier Group Code 08-50	38

Table 20 Linear Operational Amplifier Group Code 08-50	38

Table 21 Memory SRAM Group Code 08-20	38

Table 22 Memory EEPROM Group Code 08-24	38

Table 23 Switching Regulator Group Code 08-54	38

Table 24 PWM Controller Group Code 08-54	39

Table 25 Counter/Divider Group Code 08-54	39

Introduction

Purpose

This document reports data and analysis for the part stress analysis of the MEPS unit of the Lagrange L5 In-Situ Consortium

Scope

This document applies to the current design of the electronics for MEPS.

The circuits analyzed in this document can be separated according to engineering and scientific purpose. For example, the power circuits are engineering in purpose, and the sensor preamp and shaper circuits are scientific in purpose. This analysis is confined to engineering circuits because of the specialized nature of the scientific circuits, some of which are ‘tuned’ before population but after design, and some of which involve very small currents with low stress on circuit components.

Project outline

The MEPS instrument is a particle instrument measuring the energy spectra and angular distributions of energetic electrons (30-600 keV) and ions (30-6000 keV/nuc). The full experiment consists of one unit which provides a double-ended telescope pairs (sensor heads) with four view cones per pair, two dedicated for electrons and two for ions. Utilizing any spin of the spacecraft, MEPS observations cover the full sky with a total of eight electron and eight ion view cones. As the L5 S/C is not a spinner, MEPS still provides crucial pitch-angle information to determine whether the event is scatter free. This is an important indicator for interpreting the timing information of the space weather event. Figure 1 shows a single MEPS unit, with eight view cones.

MEPS-B

Sensor

MEPS-A

Sensor

Electronics

Module

Figure 1: MEPS single unit CAD-study with two sensor heads, each of which has two double-ended telescopes. One MEPS unit provides eight view cones in total, four for electrons and four for ions.

Units: 1

Sensors: 2 per unit

FoVs: 4 double FoVs per unit. The FOVs for each sensor are separated by 70°

Angular resolution: 45° full angle

Energy range:

- 30keV/n-6MeV/n for the 20-300-300 um detector stack solution

Volume: 300x160x175 mm3 (including potential radiators and MLI)

Power: 5W for 1 unit

Mass: 2.4 kg for 1 unit

Data Products: Particle VDF

Cadence: 5(1) minutes

TRL: 6 (most parts 8-9)

Table 1: Proposed MEPS instrument heritage of MEPS from EPT Solar Orbiter Technical Parameters

The two ends of a telescope observe electrons and ions, respectively. Each telescope consists of a stack of three solid-state detectors (Figure 2). On one end, the outer detector is covered with a thin polyimide layer or parylene foil (TBC), stopping ions below a few hundred keV/nuc but letting electrons pass almost unaffected. This uppermost detector (300 μm thick Si) on this side is operated in anticoincidence with the second (middle) detector and thus observes the energy spectrum of stopping electrons. The other end of the telescope has a broom magnet (instead of a PI layer) that defects electrons below a few hundred keV (see Figure 2 for electron deflection measured with the Solar Orbiter EPT EM). This side of the telescope employs a 20 μm thick Si detector in front of the 300 μm Si detector in the middle, which thus form an ion telescope observing at energies from 30-6000 keV/nuc. Ions passing the thin first detector can be identified at high energies (MeV/nuc range) using the d E/dx vs. E technique, operable at Ekin ≥ 1 MeV/nuc for protons and alphas and ≥ 2 MeV/nuc for heavier species. Lower ion energies can nevertheless be resolved using the middle detector as an anticoincidence: Anything above 1.2 MeV energy deposit in the thin detector is heavier than protons, which in return means that the thin detector may be used as a Z ≥ 2 channel SSDs for deposited energies > 1.2 MeV (or 300 keV/nuc for He). The same logic applies for penetrating He, enabling a Z ≥ 6 channel for energies > 4.8 MeV; or 420 keV/nuc for C, 360 keV/nuc for N and 310 keV/nuc for O.

Figure 2: Working principle of an EPE double-ended telescope. The polyimide layer stops ions but lets electrons pass, whereas on the other side electrons are deflected by magnets and ions pass unaffected. The thin detector on the right side enables the separation of different ion species via the d E/dx vs. E-method (in the MeV/nuc energy range; for the separation at lower energies see text).

The permanent magnets of one sensor head (double-ended telescope pair) form a quadrupole, significantly reducing the far field. The far field is reduced even further by the other sensor head on the same unit, since its quadrupole forms an angle of 70° with respect to the first sensor head. The MEPS unit integrates the sensors and electronics in a single package. Right below the sensors in the electronic box are the preamp boards, followed by an analogue board and the back-end electronics (digital and LVPS-board[s]). Pre-flight ground energy calibration (1 % level) of all detector elements, on-axis active area calibration, selected off-axis directions active area calibration and dead-time calibration will be performed. Calibration quality will be monitored in-flight using measured data and cross-calibration with suprathermal electron/ion instruments.

Figure 3: Electron Proton Telescope on Solar Orbiter (EPT) EM measurements with a Bi- 207 source. The conversion electron lines at 482 keV and 554 keV are seen on the PI-side (Det 1) but vanish on the magnet-side (Det 2).

Details of MEPS

Figure 4 shows the LGR In-situ suite star configuration.

Figure 4 MEPS as part of the Lagrange L5 In-situ Consortium

Unit level details of MEPS

The sensors MEPS-A and MEPS-B share an Electronics Box and are therefore analyzed together as follows.

| Unit | Board |
| --- | --- |
| MEPS-A | MEPS-A Preamp |
| MEPS-B | MEPS-B Preamp |
| Ebox | MEPS Analog ADC Shaper |
| Ebox | MEPS Digital |
| Ebox | MEPS Power (LVPS) |

Figure 5 MEPS block diagram

GLOSARY AND DEFINITIONS

Acronyms and Abbreviations

| CME | Coronal Mass Ejection |
| --- | --- |
| DOCS | Deep-space Optical Communication System |
| DPU | Data Processing Unit |
| EDAC | Error Detection and Correction |
| EEE | Electrical, Electronic and Electro-mechanical |
| EMC | Electro-Magnetic Cleanliness |
| EOL | End of Life |
| EPT | Electron Proton Telescope |
| HV | High Voltage |
| IMF | Interplanetary Magnetic Field |
| LCL | Latch Current Limiter |
| LET | Linear Energy Transfer |
| LGR | Lagrange |
| MAG | Magnetometer |
| MCP | Microchannel Plate |
| MEPS | Medium Energy Particle Spectrometer |
| MICD | Mechanical Interface Drawing |
| MRAM | Magneto-Resistive Memory |
| NGRM | Next Generation Radiation Monitor |
| PLA | Plasma Analyser |
| RADEM | Radiation Monitor |
| RAM | Random Access Memory |
| RAMS | Reliability, Availability, Maintainability, Safety |
| RM | Radiation Monitor |
| SEE | Single Event Effects |
| SEP | Solar Energetic Particles |
| SIR | Stream Interaction Region |
| SIXS | Solar Intensity X-ray and particle Spectrometer |
| SO | Solar Orbiter |
| SRAM | Static Random Access Memory |
| SSA | Space Situational Awareness |
| IS | Solar Wind Analyser |
| TC | Telecommand |
| TID | Total Ionizing Dose |
| TM | Telemetry |
| TOF | Time of Flight |
| XFM | X-ray Flux Monitor |

APPLICABLE AND REFERENCE DOCUMENTS (from Solar Orbiter)

| ID. | Title | Reference | Iss./Rev. | Date |
| --- | --- | --- | --- | --- |
| AD-01 | Product Assurance Requirements for instruments | SOL-EST-RS-1937 | 2 / 1 | 2011-04-15 |
| AD-02 | EID-B | SO-EPD-EID-00001 | 2 / 4 draft | 2011-03-18 |
| AD-03 | EID-A | SOL.EST.RCD.0050 | 4 | 2011-02-28 |
| AD-04 | Science Performance Document | SO-EPD-PO-RP-0002 | 2 / 1 | 2014-05-23 |
| AD-05 | DRD for SO Payload | SOL.EST.LI.2241 | 1 / 0 | 2011-10-15 |
| AD-06 | EPT-HET and STEP Circuit Diagrams | SO-EPD-KIE-CD-0010 | 2 / 0 | 2013-10-08 |
| AD-07 | EPT-HET and STEP Block Diagrams | SO-EPD-KIE-BD-0010 | 2 / 0 | 2013-10-08 |
| AD-08 | EPT-HET and STEP EEE List | SO-EPD-KIE-LI-0050 | 1 / 3 | 2014-06-13 |

Table 2 Applicable documents

| ID. | Title | Reference | Iss. / Rev. | Date |
| --- | --- | --- | --- | --- |
| ND-01 | Space product assurance: Dependability | ECSS-Q-ST-30C | C | 2009-03-06 |
| ND-02 | Space product assurance: Derating EEE components | ECSS-Q-ST-30-11C | C | 2011-10-04 |
| ND-03 | JPL Reliability Analyses Handbook | JPL D-5703 |  | 1990-07 |

Table 3 Normative documents

| ID. | Title | Reference | Iss. / Rev. | Date |
| --- | --- | --- | --- | --- |
| RD-01 |  |  |  |  |

Table 4 Reference documents

PSA requirements

Applicable Dependability Requirements

The PSA is called for in AD-01 (RAMS 250), with reference to

ND-01 §6.4.2.8 to assure part stress is compliant with derating rules, and

ND-02 the ECSS reference for part derating rules,

and is a Lagrange L5 deliverable (AD-03 §12.8). The PSA shall be performed with the highest stresses compatible with operational conditions (AD-05 §3.46) and be compared with part derating rules (ND-02).

Circuit breakdown

Block diagrams summarize the circuit structure.

MEPS

Figure 6 MEPS circuit block diagram

The boards are delineated approximately on the block diagram, but we refer to board level block diagrams to indicate subcircuits as analysed.

Figure 7 MEPS LVPS board circuit breakdown

Figure 8 MEPS Digital board circuit breakdown

Figure 9 MEPS Analog board circuit breakdown

Figure 10 circuit breakdown MEPS-A Preamp and MEPS-B Preamp board

SPICE models

The following table provides details of the correspondence of block diagram units and board level implementation.

| EH ref | Board | Assemblies | SPICE model |
| --- | --- | --- | --- |
| A1.00 | LVPS |  |  |
| A1.01 | LVPS | Input EMI | Emi-filter |
| A1.02 | LVPS | Flyback Converter | PWM-controller-isl78841 |
| A1.03 | LVPS | RF Transformer | Solo-lvps-rf-transformer |
| A1.04 | LVPS | Power transformer |  |
| A1.05 | LVPS | ISR ANA ADC 3.3V |  |
| A1.06 | LVPS | ISR DIG FPGA1 1.5V |  |
| A1.07 | LVPS | ISR DIG FPGA2 1.5V |  |
| A1.08 | LVPS | ISR DIG 1.8V |  |
| A1.09 | LVPS | ISR DIG 3.3V |  |
| A1.10 | LVPS | LDO ANA +6.0V | Solo-positive-and-negative-ldo |
| A1.11 | LVPS | LDO ANA -6.0V | Solo-positive-and-negative-ldo |
| A1.12 | LVPS | Bias LRCS | Solo-bias-currentsource-and-monitor_step (1) |
|  |  |  | Solo-bias-currentsource-and-monitor_step (1) |
| 2.00 | DIG |  |  |
| 2.01 | DIG | LVDS I/F |  |
| 2.02 | DIG | Clock |  |
| 2.03 | DIG | LDOs SHA +-5.0V | Solo-positive-and-negative-ld |
| 3.00 | ANA |  |  |
| 3.01 | ANA | ADC shaper & HK |  |
| 3.02 | ANA | Shaper |  |
| 4.00 | EPT PA |  |  |
| 4.01 | EPT PA | Preamps |  |
| 4.02 | EPT PA | LDO PA | Solo-positive-and-negative-ldo |
| 4.03 | EPT PA | LDO ref, fet | Solo-positive-and-negative-ldo |
| 4.04 | EPT PA | CM Filter |  |
| 5.00 | HET PA |  |  |
| 5.01 | HET PA | Preamps |  |
| 5.02 | HET PA | LDO PA | Solo-positive-and-negative-ldo |
| 5.03 | HET PA | LDO ref, fet | Solo-positive-and-negative-ldo |
| 5.04 | HET PA | CM Filter |  |

(1) or solo-bias-improved-monitor_step_100krad

Part Stress

For the purpose of analysis the LVPS has been broken into the assemblies listed in the previous table.

EMI filter

| S.No | Part No in layout | Part Number (generic) | Rated parameter | Derated value | Designed value | Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | R201, R205 | 400102701R560F (SMP-PW 2010 0.56R 1% 50ppm) | Voltage: 200V Power: 2W | 160V 1W | < 112mV < 0.022mW PEAK of >50W for 1ms | 0.22 |
| 2 | R202, R204 | 40010260124R9F4 (CHRP0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 0V - 28V @ startup 0mW - 31W for 0.25μs | 0.7 |
| 3 | R203 | 4001026011004G6 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 28V <1mW | 0.22 0.2 |
| 4 | C201, C202, C206, C207 | CNC12 02SC 103KL (CNC12) | Voltage: 200V Temp: 125°C | 120V <125°C | 28V +85°C | 0.23 0.68 |
| 5 | C203, C204, C205 | 300103809C825KE | Voltage: 200V Temp: 125°C | 120V <125°C | 28V +85°C | 0.23 0.68 |
| 6 | L204 | HCESC 10 M47 1S | Current: 0.4A Temp: 125°C | ~ 100°C | <200mA 85°C | ~ 0.85 |
| 7 | L201, | SESI 9.1 10K 2WR | Current: 1.85A Temp: 125°C | ~ 100°C | <200mA 85°C | ~ 0.85 |
| 8 | L202, L203, L205, L206 | ESI 7 8K4 1S | Current: 0.6A Temp: 125°C | ~ 100°C | <200mA 85°C | ~ 0.85 |
| 9 | D201, D202 | 1N5806U02A | Ifw: 2.5A Rev Volt: 150V Tjmax: 175C | 1.25A 112.5V 110C | <200mA 0V 85C | 0.16 ~ 0,77 |

Inrush circuit

| S.No | Part No in layout | Part Number (generic) | Rated parameter | Derated value | Designed value | Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | C305, C306, C311 | 300103809C395KE (CNC54) | Voltage: 100V Temp: 125°C | 60V 125°C | 28V 85°C | 0.46 0.68 |
| 2 | C302, C303, C304, C307, C308, C309, C310, | 300103809C825KE | Voltage: 100V Temp: 125°C | 60V 125°C | 28V 85°C | 0.46 0.68 |
| 3.1 changed | C301, C314, C316, C317 | 300900807C104KC | Voltage: 50V Temp: 125°C | 30V 125°C | <1V, 12V, <12V,<12V 85°C | 0.03, 0.4 0.68 |
| 3.2 changed | C315 | 301200403C336 KD0300 | Voltage: 20V Temp: 125°C | 12V 125°C | 12V 85°C | 1 0.68 |
| 4 | R301, R304 | 4001026031000F4 (CHPHR1206) | Voltage: 300V Power: 0.25W | 240V 0.125W | 34mV, <1mW 0V,0W - <10mW for <20ms @ startup | 0.08 |
| 5 | R303 | 4001026013322F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 16V 26mW | 0.4 0.52 |
| 6 | Q301 | JANSR2N7616UB | VDS: 60V VGS: 10V IDS: 0.5A Power: 0.6W Tjmax: 150°C Rad: 100kRad (Si) | 48V 7.5V 0.375A 0.39W 110°C 100kRad | 16V 0.3V 0mA 0mW 85°C 100kRad | 0.33 0.04 ~ ~ 0.77 100KRad |
| 7 | D302 | JANS1N4467US | Izsm nonrep: 119mA Izsm rep: 2.5A Power diss: 1.5W Tjmax: 175C | 71.4mA 0.96A 0.975W 110°C | 1.6mA No surge current 19.2mW 85°C | 0.022 ~ 0.02 0.77 |
| 8 | D303 | 1N5806U02A | Ifw: 2.5A Rev Volt: 150V Tjmax: 175C | 1.25A 112.5V 110°C | <10mA 0V 85°C | 0.008 ~ 0.77 |
| 9 | L301 | ESI 7 8K4 1S | Current: 0.6A Temp: 125 | ~ 100 | <200mA 85 | ~ 0.85 |

5V Regulator

| S.No | Part No in layout | Part Number (generic) | Rated parameter | Derated value | Designed value | Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Q302 | SOC5551SW (2N5551) | VCEO: 160V VCBO: 180V VEBO: 6V IC: 0.5A Power: 0.36W Tjmax: 200C Rad: 100kRad(Si) | 120V 135V 4.5V 0.365A 0.234W 110°C 100kRad | <7V <7V 0V <100mA <136mW 85°C 100kRad | 0.06 0.05 ~ 0.27 0.58 0.77 100kRad |
| 2 | D306 | JANS1N6491US (1N6491) | Izsm nonrep: 255mA Izsm rep: 2.5A Power diss: 1.5W Tjmax: 175C | 204mA 1.625A 0.975W 110°C | <21mA 0A <118mW 85°C | 0.1 ~ 0.12 0.77 |
| 3 | R312 | 40010260110R0F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <0.13V <2mW | 0.003 0.04 |
| 4 | R307, R308, R309 | 4001026021820F4 (CHPHR0805) | Voltage: 100V Power: 0.2W | 80V 0.1W | <7V <68mW | 0.087 0.68 |
| 5 | C320, C326 | 300103706C475KA | Voltage: 25V Temp: 125°C | 15V 125°C | <12V, 5V 85°C | 0.8, 0.33 0.68 |
| 6 | C321, C322, C324, C329 | 300900807C104KC | Voltage: 50V Temp: 125°C | 30V 125°C | 12V,<12V, 5.6V,5V 85°C | 0.4, 0.033, 0.16 0.68 |

Switching regulator: ISR FPGA 1.5V

| S.No | Part No in layout | Part Number (generic) | Rated parameter | Derated value | Designed value | Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | IC401,IC402 | MSK5059RHG | Vin: 16V Iout: 4.5A Tjmax: 150°C | 14.4V 3.6A 110°C | 6.7V 85°C | 0.46 0.77 |
| 2 | D401, D404 | 1N6642U02D | Ifw: 0.3A Rev Volt: 100V Tjmax: 175C | 0.15A 75V 110°C | <10mA <5V 85°C | <0.06 <0.06 0.77 |
| 3 | D403, D405 | 1N5822U02B | Ifw: 3A Rev Volt: 40V Tjmax: 150°C | 1.5A 30V 110°C | <500mA <7V 85°C | <0.33 <0.23 0.77 |
| 4 | L402, L404 | SESI 9.1 10K 2WR | Current: 1.85A Temp: 125°C | ~ 100°C | <1A 85°C | ~ 0.85 |
| 5 | L403, L405 | ESI 7 1K2 1S | Current: 1.6A Temp: 125°C | ~ 100°C | <1A 85°C | ~ 0.85 |
| 6 | C402, C415 | 300103706C106KX | Voltage: 16V Temp: 125°C | 9.6V 125°C | 6.7V 85°C | 0.7 0.68 |
| 7 | C401, C414 | 300900906C474KX | Voltage: 16V Temp: 125°C | 9.6V 125°C | 6.7V 85°C | 0.7 0.68 |
| 8 | C408, C409, C410, C417, C418, C419 | 301200405C157KC0040 | Voltage: 16V@85°C Ripple: 94mV@85°C Temp: 125°C | 9.6V 94mV 125°C | 1.5V 2mV, <1mV, <1mV, 2mV 85°C | 0.16 0.02, <0.01 0.68 |
| 9 | C407, C416 | 300103708C476KX (CNC34) | Voltage: 16V Temp: 125°C | 9.6V 125°C | 1.5V 85°C | 0.16 0.68 |
| 10 | C412,C421 | 300900306C1501FC | Voltage: 50V Temp: 125°C | 30V 125°C | 1.5V 85°C | 0.05 0.68 |
| 11 | C411, C420 | 300903706C1001FX | Voltage: 16V Temp: 125°C | 9.6V 125°C | <1V 85°C | <0.1 0.68 |
| 12 | R402, , R407 | 4001026011001F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <3V <2mW | <0.075 <0.04 |
| 13 | R404, R409 | 4001026011002F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <2V <2mW | <0.05 <0.04 |
| 14 | R401, R406 | 400102309R1001B2 (PFRR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <2V <2mW | <0.05 <0.04 |
| 15 | R403, R408 | 400102309R4221B2 (PFRR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <3V <2mW | <0.075 <0.04 |

Switching regulator: ISR SRAM 1.8V

| S.No | Part No in layout | Part Number (generic) | Rated parameter | Derated value | Designed value | Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | IC404 | MSK5059RHG | Vin: 16V Iout: 4.5A Tjmax: 150°C | 14.4V 3.6A 110°C | 6.7V 85°C | 0.46 0.77 |
| 2 | D408 | 1N6642U02D | Ifw: 0.3A Rev Volt: 100V Tjmax: 175C | 0.15A 75V 110°C | <10mA <5V 85°C | <0.06 <0.06 0.77 |
| 3 | D409 | 1N5822U02B | Ifw: 3A Rev Volt: 40V Tjmax: 150°C | 1.5A 30V 110°C | <500mA <7V 85°C | <0.33 <0.23 0.77 |
| 4 | L408 | SESI 9.1 10K 2WR | Current: 1.85A Temp: 125°C | ~ 100°C | <0.15A 85°C | ~ 0.85 |
| 5 | L409 | ESI 7 8K4 1S | Current: 0.6A Temp: 125°C | ~ 100°C | <0.15A 85°C | ~ 0.85 |
| 6 | C430 | 300103706C106KX | Voltage: 16V Temp: 125°C | 9.6V 125°C | 6.7V 85°C | 0.7 0.68 |
| 7 | C427 | 300900906C474KX | Voltage: 16V Temp: 125°C | 9.6V 125°C | 6.7V 85°C | 0.7 0.68 |
| 8 | C431, C432 | 301200405C157KC0040 | Voltage: 16V@85°C Ripple: 94mV@85°C Temp: 125°C | 9.6V 94mV 125°C | 1.8V 2mV, <1mV 85°C | 0.187 0.02, <0.01 0.68 |
| 9 | C434 | 300900306C1501FC | Voltage: 50V Temp: 125°C | 30V 125°C | 1.8V 85°C | 0.06 0.68 |
| 10 | C433 | 300903706C1001FX | Voltage: 16V Temp: 125°C | 9.6V 125°C | <1V 85°C | <0.1 0.68 |
| 11 | R415 | 4001026011001F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <3V <2mW | <0.075 <0.04 |
| 12 | R417 | 4001026011002F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <2V <2mW | <0.05 <0.04 |
| 13 | R414 | 400102309R1581B2 (PFRR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <2V <2mW | <0.05 <0.04 |
| 14 | R416 | 400102309R3241B2 (PFRR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <3V <2mW | <0.075 <0.04 |

Switching regulator: ISR DIG 3.3V

| S.No | Part No in layout | Part Number (generic) | Rated parameter | Derated value | Designed value | Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | IC403 | MSK5059RHG | Vin: 16V Iout: 4.5A Tjmax: 150°C | 14.4V 3.6A 110°C | 6.7V 85°C | 0.46 0.77 |
| 2 | D406 | 1N6642U02D | Ifw: 0.3A Rev Volt: 100V Tjmax: 175C | 0.15A 75V 110°C | <10mA <5V 85°C | <0.06 <0.06 0.77 |
| 3 | D407 | 1N5822U02B | Ifw: 3A Rev Volt: 40V Tjmax: 150°C | 1.5A 30V 110°C | <500mA <7V 85°C | <0.33 <0.23 0.77 |
| 4 | L406 | SESI 9.1 10K 2WR | Current: 1.85A Temp: 125°C | ~ 100°C | <0.2A 85°C | ~ 0.85 |
| 5 | L407 | ESI 7 8K4 1S | Current: 0.6A Temp: 125°C | ~ 100°C | <0.2A 85°C | ~ 0.85 |
| 6 | C423 | 300103706C106KX | Voltage: 16V Temp: 125°C | 9.6V 125°C | 6.7V 85°C | 0.73 0.68 |
| 7 | C422 | 300900906C474KX | Voltage: 16V Temp: 125°C | 9.6V 125°C | 6.7V 85°C | 0.73 0.68 |
| 8 | C424, C425, C426 | 301200405C157KC0040 | Voltage: 16V@85°C Ripple: 94mV@85°C Temp: 125°C | 9.6V 94mV 125°C | 3.3V 2mV, <1mV 85°C | 0.7 0.02, <0.01 0.68 |
| 9 | C429 | 300900306C1501FC | Voltage: 50V Temp: 125°C | 30V 125°C | 3.3V 85°C | 0.11 0.68 |
| 10 | C428 | 300903706C1001FX | Voltage: 16V Temp: 125°C | 9.6V 125°C | <1V 85°C | <0.1 0.68 |
| 11 | R411 | 4001026011001F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <3V <2mW | <0.075 <0.04 |
| 12 | R413 | 4001026011002F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <2V <2mW | <0.05 <0.04 |
| 13 | R412 | 400102309R1621B2 (PFRR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <2V <2mW | <0.05 <0.04 |
| 14 | R410 | 400102309R2801B2 (PFRR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <3V <2mW | <0.075 <0.04 |

Switching regulator: ISR ANA ADC 3.3V

| S.No | Part No in layout | Part Number (generic) | Rated parameter | Derated value | Designed value | Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | IC601 | MSK5059RHG | Vin: 16V Iout: 4.5A Tjmax: 150°C | 14.4V 3.6A 110°C | 6.7V 85°C | 0.46 0.77 |
| 2 | D601 | 1N6642U02D | Ifw: 0.3A Rev Volt: 100V Tjmax: 175C | 0.15A 75V 110°C | <10mA <5V 85°C | <0.06 <0.06 0.77 |
| 3 | D602 | 1N5822U02B | Ifw: 3A Rev Volt: 40V Tjmax: 150°C | 1.5A 30V 110°C | <500mA <7V 85°C | <0.33 <0.23 0.77 |
| 4 | L601, L602, L603 | ESI 7 8K4 1S | Current: 0.6A Temp: 125°C | ~ 100°C | <0.05A 85°C | ~ 0.85 |
| 5 | C602 | 300103706C106KX | Voltage: 16V Temp: 125°C | 9.6V 125°C | 6.7V 85°C | 0.7 0.68 |
| 6 | C601 | 300900906C474KX | Voltage: 16V Temp: 125°C | 9.6V 125°C | 6.7V 85°C | 0.7 0.68 |
| 7 | C604, C605, C606 | 301200405C157KC0040 | Voltage: 16V@85°C Ripple: 94mV@85°C Temp: 125°C | 9.6V 94mV 125°C | 3.3V 2mV, <1mV, <1mV 85°C | 0.7 0.02, <0.01 0.68 |
| 8 | C609 | 300900306C1501FC | Voltage: 50V Temp: 125°C | 30V 125°C | 3.3V 85°C | 0.11 0.68 |
| 9 | C608 | 300903706C1001FX | Voltage: 16V Temp: 125°C | 9.6V 125°C | <1V 85°C | <0.1 0.68 |
| 10 | C603 | 300900807C104KC | Voltage: 50V Temp: 125°C | 30V 125°C | 3.3V 85°C | 0.075 0.68 |
| 11 | C607 | 301200402C226KC0600 | Voltage: 16V@85°C Ripple: 94mV@85°C Temp: 125°C | 9.6V 94mV 125°C | 3.3V <1mV 85°C | 0.34 <0.01 0.68 |
| 12 | R602 | 4001026011001F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <3V <2mW | <0.075 <0.04 |
| 13 | R604 | 4001026011002F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <2V <2mW | <0.05 <0.04 |
| 14 | R603 | 400102309R1331B2 (PFRR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <2V <2mW | <0.05 <0.04 |
| 15 | R601 | 400102309R2321B2 (PFRR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <3V <2mW | <0.075 <0.04 |

Positive and Negative LDOs

| S.No | Part No in layout | Part Number (generic) | Rated parameter | Derated value | Designed value | Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | IC603A, IC603B | RH1498MW | VCC: 36V VIN: 15V Iout:1.8mA Tjmax: 150°C | 32.4V 10.5V 1.44mA 110°C | 13.4V 2.5V, 0V 0mA 85°C | 0.41 0.24 0.77 |
| 2 | IC602 | 5962F0052301VXC (IS-1009) | Ifw: 10mA Vrev: 2.5V reference Tjmax: 175C | 7.5mA n/a 110°C | 3.6mA ~ 85°C | 0.48 ~ 0.77 |
| 3 | Q601 | JANSR2N7626UB | VDS: 60V VGS: 10V IDS: 0.33A Power: 0.57W Tjmax: 150°C Rad: 100kRad(Si) | 48V 7.5V 0.2475A 0.37W 110°C 100kRad | 0.7V, 6.7V @ Startup <2V, 4V spike @ Startup <200mA <140mW 85°C 100kRad | 0.11 <0.26, 0.53 0.38 0.77 100kRad |
| 4 | Q602 | JANSR2N7616UB | VDS: 60V VGS: 10V IDS: 0.5A Power: 0.6W Tjmax: 150°C Rad: 100kRad(Si) | 48V 7.5V 0.375A 0.39W 110°C 100kRad | 0.7V, 6.7V @ startup <2V, 2V spike @ startup <50mA <35mW 85°C 100kRad | 0.11 <0.26, 0.26 0.133 0.89 0.77 100kRad |
| 5 | D604, D605 | 1N6642U02D | Ifw: 0.3A Rev Volt: 100V Tjmax: 175C | 0.15A 75V 110°C | 0mA 4.7V 85°C | ~ 0.063 0.77 |
| 6 | L606, L608 | HRB0603S102P.200FT | Current: 0.2A Temp: 125°C | ~ 100°C | <3mA 85°C | ~ 0.85 |
| 7 | L604, L609 | ESI 7 8K4 1S | Current: 0.6A Temp: 125°C | ~ 100°C | 200mA, 50mA 85°C | ~ 0.85 |
| 8 | C610, C614, C615, C620, C627, C630, C632, | 300900807C104KC | Voltage: 50V Temp: 125°C | 30V 125°C | <7V 85°C | 0.233 0.68 |
| 9 | C611, C619, C622, C624, C628, C633 | 301200402C226KC0600 | Voltage: 16V@85°C Ripple: 94mV@85°C Temp: 125°C | 9.6V 94mV 125°C | <7V <1mV 85°C | <0.73 <0,01 0.68 |
| 10 | C612, C632 | 300903706C47C0JC | Voltage: 50V Temp: 125°C | 30V 125°C | <7V 85°C | <0.233 0.68 |
| 11 | C621, C623 | 300103706C106KX | Voltage: 16V Temp: 125°C | 9.6V 125°C | 6V 85°C | 0.625 0.68 |
| 12 | R606 | 4001026011001F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 4.2V <20mW | 0.105 0.4 |
| 13 | R608, R613 | 4001026012210F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 0V, 113mV 0W, <1mW | 0.003 0.02 |
| 14 | R609, R617 | 4001026013321F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 0V 0W | ~ ~ |
| 15 | R615 | 400102309R4021B2 (PFRR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 2.37V <2mW | 0.06 <0.04 |
| 16 | R607, R618 | 4001026011213F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 0V 0W | ~ ~ |
| 17 | R614 | 400102309R1022B2 (PFRR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 6V <4mW | 0.15 <0.08 |
| 18 | R611 | 400102309R5111B2 (PFRR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 3.5V <3mW | 0.0875 0.06 |
| 19 | R610,R616 | 40010260124R9F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <100mV <1mW | 0.0025 0.02 |

BIAS Current Source and Monitor

| S.No | Part No in layout | Part Number (generic) | Rated parameter | Derated value | Designed value | Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | IC501, IC502 | RH1056AMW | VCC: 20V VIN: 20V IOUT: 6.5mA Tjmax:125°C | 18V 14V 4.96mA 110°C | 13V - 15Vp @ startup <0.5V <100μA, <300μA 85°C | 0.83 <0.035 <0.02, <0.06 0.77 |
| 2 | IC503A | RH1498MW | VCC: 36V VIN: 15V IOUT: 1.8mA Tjmax: 150°C | 32.4V 10.5V 1.44mA 110°C | 13V - 15Vp @ startup <3V <100μA 85°C | 0.46 <0.28 <0.069 0.77 |
| 3 | Q501, Q502 | SOC5551SW (2N5551) | VCEO: 180V VCBO: 160V VEBO: 6V ICmax: 0.6A Power: 0.36W Tjmax: 200C | 135V 120V 4.5V 0.45A 0.234W 110°C | 5V - 80V @ startup 1V 500mV <10μA <1mW 85°C | 0.6 0.008 0.11 ~ 0.004 0.77 |
| 4 | D504 | JANTXV1N4486US (1N4486) | Izsm non rep: 19mA Izsm rep: 0.19A Power diss: 1.5W @ 85°C tj max :175°C | 15.2mA 0.124mA 0.975W 110°C | <10μA <10μA <1mW 85°C | 0.0006 ~ 0.001 0.77 |
| 5 | D505 | JANTXV1N3595US (1N3595) | Ifw:1nA Rev Volt: 125V Tj max: 150°C | 0.5nA 93.75V 110°C | 0mA 6V 85°C | ~ 0.064 0.77 |
| 6 | L501, L502 | HRB0603S102P.200FTEM | Current: 0.2A Temp: 125°C | ~ 100°C | 85°C | ~ 0.85 |
| 7 | C505, C506, C507 | CNC12 02SC 103KL | Voltage: 500V Temp: 125°C | 300V 125°C | 75V 85°C | 0.25 0.68 |
| 8 | C509 | 300903806C103JC | Voltage: 50V Temp: 125°C | 30V 125°C | 0.6V 85°C | 0.02 0.68 |
| 9 | C510, C513, C514, C516, C518, C519, C520, C521, C522, C523 | 300900807C104KC | Voltage: 50V Temp: 125°C | 30V 125°C | 0V, 6.5V, <0.5V, <3.3V, 6.5V, 6.5V, 6.5V, <3.3V, 6.5V, 6.5V 85°C | 0.22, <0.11, <0.016 0.68 |
| 10 | C512, C515 | 300903706C1000JC | Voltage: 50V Temp: 125°C | 30V 125°C | <3V 85°C | <0.1 0.68 |
| 11 | C504 | 300103811C685KG | Voltage: 200V Temp: 125°C | 120V 125°C | 75V 85°C | 0.625 0.68 |
| 12 | R505, R511 | 4001026011002F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 0.6V, 0V <1mW, 0W | 0.015 0.02 |
| 13 | R515, R525, R526 | 400102309R1002B2 (PFRR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <2V <1mW | <0.05 <0.02 |
| 14 | R502, R509 | 4001026021005G6 (CHPHR0805) | Voltage: 100V Power: 0.2W | 80V 0.1W | 5V - 80V @ startup <5mW | 1 <0.05 |
| 15 | R503, R504, R506, R513, R531 | 4001026011003F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <1V <1mW | <0.025 <0.02 |
| 16 | R507, R508, R510 | 4001026012213F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <1V <1mW | <0.025 <0.02 |
| 17 | R514 | 400102309R2742B2 (PFRR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <2V <1mW | <0.05 <0.02 |

3-Stage Voltage Multiplier

| S.No | Part No in layout | Part Number (generic) | Rated parameter | Derated value | Designed value | Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | D501, D502, D503 | 1N5806U02A | Ifw: 1A Rev Volt: 60V Tjmax: 150°C | 1.25A 112.5V 110°C | <1mA <60V 85°C | <0.0008 0.53 0.77 |
| 2 | C501, C502, C503, C508 | CNC12 02SC 103KL | Voltage: 500V Temp: 125°C | 300V 125°C | <80V 85°C | <0.26 0.68 |
| 3 | C511 | 300900807C104KC | Voltage: 50V Temp: 125°C | 30V 125°C | 0V 85°C | ~ 0.68 |
| 4 | R501 | 4001026011003F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 0.6V <1mW | 0.015 0.02 |

Planar Transformer

| S.No | Part No in layout | Part Number (generic) | Rated parameter | Derated value | Designed value | Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Tranfo | E22/6/16/R-3C96-A400-P, PLT22/16/2.5/S-3C96 (planar ferrite core 3C96) CLM-E22/PLT22 (slotted ferrite plate 3C96) | Temp: 125°C | 105C | 85°C | 0.81 |
| 2 | D305, D506, D603, D606 | 1N5819U02B | Ifw: 1A Rev. Volt.: 60V Tjmax: 150°C | 0.5A 45V 110°C | <100mA, <250mA, <200mA, ~40mA 30V, <20V, <20V, <20V 85°C | <0.2 <0.5 <0.4, 0.1 0.66, <0.44 0.77 |
| 3 | D402 | 1N5822U02B | Ifw: 3A Rev. Volt.: 40V Tjmax: 150°C | 1.5A 30V 110°C | ~350mA <20V 85°C | 0.233 <0.66 0.77 |
| 4 | L401 | ESI 7 1K2 1S | Current: 1.6A Temp: 125°C | ~ 100°C | ~350mA 85°C | ~ 0.85 |
| 5 | L605, L607 | ESI 7 8K4 1S | Current: 0.6A Temp: 125°C | ~ 100°C | <200mA, ~40mA 85°C | ~ 0.85 |
| 6 | C319 | 300103706C475KA | Voltage: 25V Temp: 125°C | 15V 125°C | 12V 85°C | 0.8 0.68 |
| 7 | C403, C404, C405, C406, C616, C617, C618, C625, C626 | 301200405C157KC0040 | Voltage: 16V Ripple: 203mV Temp: 125°C | 9.6V 203mV 125°C | 6.7V, 6.7V, 6.7V, 6.7V, 6.7V, 6.7V, 6.7V, 6.7V, 6.7V 50mV, 50mV, <2mV, <2mV, 50mV, <2mV, <2mV, 35mV, <2mV 85°C | 0.7 0.24, 0.001, 0.172 85°C |
| 8 | C524 | 301200405C476KE0065 | Voltage: 25V Ripple: 119mV Temp: 125°C | 15V 119mV 125°C | 12V <90mV 85°C | 0.8 <0.75 0.68 |
| 9 | C613, C629 | 300103706C106KX | Voltage: 16V Temp: 125°C | 9.6V 125°C | 6.7V, 6.7V 85°C | 0.7 0.68 |
| 10 | C527 | 300900807C104KC | Voltage: 50V Temp: 125°C | 30V 125°C | 0V 85°C | ~ 0.68 |
| 11 | R546 | 4001026011003F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 0V 0mW | ~ ~ |

Snubber Circuit

| S.No | Part No in layout | Part Number (generic) | Rated parameter | Derated value | Designed value | Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | D301 | 1N5806U02A | Ifw: 2.5A Rev Volt: 150V Tjmax: 175C | 1.25A 112.5V 110°C | 0.065A RMS 24V 85°C | 0.052 0.21 0.77 |
| 2 | C312 | CNC12 02SC 103KL | Voltage: 500V Temp: 125°C | 300V 125°C | 35V 85°C | 0.12 0.68 |
| 3 | R305 | 4001026034702J4 (CHPHR1206) | Voltage: 300V Power: 0.25W | 240V 0.125W | 35V <26mW | 0.145 0.2 |

PWM Modulator, Crystal Oscillator and Divider (Flyback Converter)

| S.No | Part No in layout | Part Number (generic) | Rated parameter | Derated value | Designed value | Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | IC301 | ISL78841ASEHQF | VCC: 30V Iout: 1A Tjmax: 150C | 27V 0.8A 110C | VDD: 11.5V 85C | 0.42 0.77 |
| 2 | IC302 | HCC4024BK | VCC: 22V VIN: 22.5V Power: 200mW Tjmax: 150C | 19.8V 20.25V 0.16W 110C | VDD: 5V 85C | 0.25 0.77 |
| 3 | X302 | QT188HCD10E-2.000MHz | crystal oscillator 2MHz CMOS 5.0V 100ppm |  | VDD: 5V |  |
| 4 | D307 | 1N5819U02B | Ifw: 1A Rev. Volt: 60V Tjmax: 150°C | 0.5A 45V 110°C | <60mA 12V 85°C | <0.12 0.27 0.77 |
| 5 | Q303 | IRHNJ57133SE | VDS: 130V VGS: 20V IDS:12.5A Power: 75W Tjmax: 150°C Rad: 100kRad(Si) | 104V 15V 9.375A 48.75W 110°C 100kRad(Si) | <70V <12V <2A <100mW 85°C 100kRad | <0.67 <0.8 <0.21 0.002 0.77 100kRad |
| 6 | C318, C327, C328, C331, C336 | 300900807C104KC | Voltage: 50V Temp: 125°C | 30V 125°C | 12V, 5V, 11.5V, 5V, 5V 85°C | 0.16, 0.4, 0.38 0.68 |
| 7 | C323, C330 | 300903706C2210FC | Voltage: 50V Temp: 125°C | 30V 125°C | 5V, 12V 85°C | 0.16, 0.4 0.68 |
| 8 | C325, C332, C333 | 300903706C1001FX | Voltage: 16V Temp: 125°C | 9.6V 125°C | 5V, 5V, 5V 85°C | 0.52 0.68 |
| 9 | R320 | 40010260124R9F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 0V <10mW | ~ 0.2 |
| 10 | R318 | 4001026012210F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 0V <10mV | ~ <0.2 |
| 11 | R317 | 400102701R560F (SMP PW 2010) | Voltage: 200V Power: 2W | 160V 1W | <1V <25mW | <0.00625 <0.025 |
| 12 | R311 | 4001026011003F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 5V <1mW | 0.125 <0.02 |
| 13 | R316 | 4001026016811F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 5V <4mW | 0.125 <0.08 |
| 14 | R319 | 40010260151R1F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 5V <20mW | 0.125 <0.4 |
| 15 | R313 | 40010260110R0F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <12mV <1mW | 0.3/1000 <0.02 |
| 16 | R315 | 4001026011001F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 0V <1mW | ~ <0.02 |
| 17 | R306 | 400102309R3832B2 (PFRR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 9.5V <3mW | 0.24 <0.06 |
| 18 | R310 | 400102309R1002B2 (PFRR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 2.5V <1mW | 0.0625 0.02 |
| 19 | R314 | 4001026011002F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 12V <15mW | 0.3 <0.3 |

RF Transformer

| S.No | Part No in layout | Part Number (generic) | Rated parameter | Derated value | Designed value | Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | D308 | 1N5819U02B | Ifw: 1A Rev. Volt: 60V Tjmax: 150°C | 0.5A 45V 110°C | <10mA <10V 85°C | <0.02 <0.22 0.77 |
| 2 | L302 | HCESC 10 M47 1S | Current: 0.4A Temp: 125°C | ~ 100°C | <200mA 85°C | ~ 0.85 |
| 3 | C334, C335 | 300903806C103JC | Voltage: 50V Temp: 125°C | 30V 125°C | <10V 85°C | <0.33 0.68 |
| 4 | R321, R322 | 4001026012210F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <2V <20mW peak | <0.05 <0.04 |
| 5 | R323 | 4001026011003F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | <1V <10mW | <0.025 <0.2 |
| 6 | R324 | 4001026011004G6 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 1.5V-3V peak ~0W | 0.075 ~ |

PWM Op Heater

| S.No | Part No in layout | Part Number (generic) | Rated parameter | Derated value | Designed value | Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | R517, R518, R519, R520, R521, R522, R523, R524, R527, R528, R529, R530, R532, R533, R534, R535, R536, R537, R538, R539, R540, R541, R542, R543 | 40010260175R0F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 3V 120mW | 0.075 |
| 2 | Q503, Q504, Q505 | IRHLUB770Z4 | VDS: 60V VGS: 10V IDS: 0.5A Power: 0.6W Tjmax: 150°C Rad: 100kRad(Si) | 48V 7.5V 0.375V 0.39W 110°C 100kRad(Si) | 12V 3.3V 80mA 240mW 85°C 100kRad | 0.25 0.44 0.21 0.61 0.77 100kRad |
| 3 | R544, R547 | 4001026011003F4 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 0V 0W | ~ ~ |
| 4 | R545 | 4001026011004G6 (CHPHR0603) | Voltage: 50V Power: 0.1W | 40V 0.05W | 3.3V <1mW | 0.082 0.02 |
| 5 | C525, C526 | 300900807C104KC | Voltage: 50V Temp: 125°C | 30V 125°C | 3.3V 85°C | 0.11 0.68 |

Summary

No stress in excess of the derated values were found for passive elements in the above circuits.

For active elements, not all parameters in [ND-02] were applicable, but of those analyzed, no excess stresses were found.

Derating analysis

The following tables list the individual groups according to the ECSS-Q-ST-30-11C Rev1. The thermal design of the LVPS was based on the goal to reach a maximum temperature of 85°C for all the components, and the derating analysis is based on the design goal.

Table 5 Ceramic capacitors Group Code 01-01

| Generic Part Type | Manf | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| 300903706C1001FX 300900306C2671FX 300103706C106KX 300900807C224KX 300900906C474KX | Eurofarad | Voltage: 16V Temperature: 125°C | 0.6 <1 | 9.6V | Timing capacitor PWM, filter, LDO o/p, flyback converter secondary o/p, boost capacitors for secondary switching regulator. |
| 300103706C475KA | Eurofarad | Voltage: 25V Temperature: 125°C | 0.6 <1 | 15V <125°C | Flyback converter secondary o/p |
| 300900807C104KC 300900306C1501FC 300903806C103JC 300903806C682KC 300903706C4750FC 300903706C4320FC 300903706C3920FC 300903706C3650FC 300903706C3320FC 300903706C3300JC 300903706C2210FC 300903706C1000JC 300903706C47C0JC 300903706C33C0JC 300903706C10C0FC 300903706C3C30FC 300903706C1C00CC | Eurofarad | Voltage: 50V Temperature: 125°C | 0.6 <1 | 30V <125°C | Shaper capacitor, Filter Capacitor analog LDO's, Preamp feedback, cap for GND filters, Filter Capacitor linear regulator |
| 300103809C825KE 300103809C395KE | Eurofarad | Voltage: 100V Temperature: 125°C | 0.6 <1 | 60V <125°C | EMI filter & flyback converter primary input capacitors |
| 300103811C685KG | Eurofarad | Voltage: 200V Temperature: 125°C | 0.6 <1 | 120V <125°C | Bias low ramp current source capacitor |
| CN1202SC103KL | Eurofarad | Voltage: 500V Temperature: 125°C | 0.6 <1 | 300V <125°C | Bias cascade, Bias filter capacitor, EMI filter |

Table 6 Solid Tantalum capacitors Group Code 01-04

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| 301200405C157KC0040 | AVX (TES) | Voltage: 16V@85°C Ripple: 94mV@85°C Temperature: 85°C | 0.6 1 <1 | 9.6V 94mV <125°C | Flyback sec. Output tantalum capacitor +/-6VANA, 6VDIG |
| 301200402C226KC0600 | AVX (TES) | Voltage: 16V@85°C Ripple: 203mV@85°C Temperature: 85°C | 0.6 1 <1 | 9.6V 203mV <125°C | Main power line blocking capacitor analog board |
| 301200405C476KE0065 | AVX (TES) | Voltage: 25V@85°C Ripple: 119mV@85°C Temperature: 85°C | 0.6 1 <1 | 15V 119mV <125°C | Flyback sec. Output tantalum capacitor +12VHVPS, +12VSEC |

Table 7 Chip resistors (Thick & Thin film) Group Code 10-08

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| CHPHR 0603 | Vishay | Voltage: 50V Power: 0.1W | 0.8 0.5 | 40V 0.05W | passive line filter resistor, Gate MOSFET serial resistor LVPS, Various etc.., |
| CHPHR 0805 | Vishay | Voltage: 100V Power: 0.2W | 0.8 0.5 | 80V 0.1W | BIAS high voltage (<100V) resistor |
| CHPHR 1206 | Vishay | Voltage: 300V Power: 0.25W | 0.8 0.5 | 240V 0.125 | Internal heater resistor network, Snubber resistor, various etc., |
| PFRR 0603 | Vishay | Voltage: 50V Power: 0.1W | 0.8 0.5 | 40V 0.05W | Voltage divider in LDO and HK voltage measurement, Various etc.., |

Table 8 Voltage reference / Zener diode Group Code 04-04

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| 1N4486 | Microsemi | Izsm non rep: 19mA Izsm rep: 0.19A Power diss: 1.5W @ 85°C tj max :175°C | 0.8 0.65 0.65 Max-110°C | 15.2mA 0.124mA 0.975W 110°C | Z-Diode 75V for low ramp BIAS Current Source |
| 1N4482 | Microsemi | Izsm non rep: 28mA Izsm rep: 0.28A Power diss: 1.5W @ 85°C tj max :175°C | 0.8 0.65 0.65 Max-110°C | 22.4mA 0.182A 0.975W 110°C | Z-Diode 51V for low ramp BIAS current Source |
| 1N4467 | Microsemi | Izsm non rep: 119mA Izsm rep: 1.2A Power diss: 1.5W @ 85°C tj max :175°C | 0.8 0.65 0.65 Max-110°C | 95.2mA 0.78A 0.975W 110°C | LVPS simple line regulator reference for PWM regulator |
| 1N6491 | Microsemi | Izsm non rep: 255mA Izsm rep: 2.5A Power diss: 1.5W @ 85°C tj max :175°C | 0.8 0.65 0.65 Max-110°C | 204mA 1.625A 0.975W 110°C | LVPS simple line regulator reference for +5V |

Table 9 RF/Microwave Schottky Si diodes Group Code 04-05

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| 1N5822U | ST | Ifw:3A Rev. Volt: 40V Tj max: 150°C | 0.5 0.75 110°C | 1.5A 30V 110°C | Flyback DC/DC converter, Switching Regulator |
| 1N5819U | ST | Ifw:1A Rev Volt: 60V Tj max: 150°C | 0.5 0.75 110°C | 0.5A 45V 110°C | LVPS clock transformer reconstruction circuit |
| 1N5806U | ST | Ifw:2.5A Rev Volt: 150V Tj max: 175°C | 0.5 0.75 110°C | 1.25A 112.5V 110°C | BIAS voltage multiplier, flyback-converter snubber, 28V input |
| 1N6642U | ST | Ifw:0.3A Rev Volt: 100V Tj max: 175°C | 0.5 0.75 110°C | 0.15A 75V 110°C | LDO startup gate-source MOSFET protection diode |
| 1N3595 | Microsemi | Ifw:1nA Rev Volt: 125V Tj max: 150°C | 0.5 0.75 110°C | 0.5nA 93.75V 110°C | Protection diode BIAS current measurement |

Table 10 Inductor cores Group Code 07-02

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| HCESC 10 M47 1S | Exxelia | Temp: 125°C | Max: 20°C | 100°C | Common mode power line filter, clock transofromer in LVPS, instead of optocoupler and EMI-filter |
| ESI 7 1K2 1S | Exxelia | Temp: 125°C | Max: 20°C | 100°C | Power line filter |
| ESI 7 8K4 1S | Exxelia | Temp: 125°C | Max: 20°C | 100°C | EMI Filter |
| SESI 9.1 10K 2WR | Exxelia | Temp: 125°C | Max: 20°C | 100°C | Power Inductor for switching regulator |
| HRB0603S102P.200FT | AEM | Temp: 125°C | Max: 20°C | 100°C | Ferrite bead to filter power lines for dedicated parts |

Table 11 Bipolar PNP Transistor Group Code 12-01

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| 2N5401 SOC5401SW LCC-3 | STMicroelectronics | VCEO : 160V. VCBO : 150V. VEBO : 5 V. IC : 0.5A. Power:0.36 W. T jmax : 200°C. Radiation: 100kRad(Si). | 0.75. 0.75. 0.75. 0.75. 0.65. 110°C. 1. | 120 V. 112.5 V. 3.75 V. 0.375 A. 0.234 W. 110°C. | 150V PNP transistor for STEIN BIAS current source to prevent damage ASIC inputs. |

Table 12 JFET N-Channel Group Code 12-15

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| BF862 BF862 SOT23-3 | NXP Semiconductors | VDS : 20V. VGS : 20V. VGD : 20V. IDS : 40mA. Power:300mW. T jmax : 150°C. Radiation:. 100kRad(Si). | 0.75. 0.75. 0.75. 0.75. 0.80. 110°C. 1. | 15 V. 15 V. 15 V. 30mA. 240mW. 110°C. 1. | JFET for preamp with best noise at lowest input capacitance |

Table 13 FET P-Channel Group Code 12-06

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| IRHLUB7970Z4 JANSR2N7626UB | International Rectifier | VDS : 60V. VGS : 10V. IDS : 0.33A. Power:0.57 W. T jmax : 150°C. Radiation: 100kRad(Si). | 0.80. 0.75. 0.75. 0.65. 110°C. 1. | 48 V. 7.5 V. 0.2475A. 0.37 W. 110°C. 1. | Discrete LDO |

Table 14 FET N-Channel Group Code 12-06

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| IRHLUB770Z4 JANSR2N7616UB | International Rectifier | VDS : 60V. VGS : 10V. IDS : 0.5A. Power:0.6 W. T jmax : 150°C. Radiation: 100kRad(Si). | 0.80. 0.75. 0.75. 0.65. 110°C. 1. | 48 V. 7.5 V. 0.375A. 0.39 W. 110°C. 1. | Discrete LDO, linear startup regulator for PWM |

Table 15 FET N-Channel Group Code 12-06

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| IRHNJ57133SE JANSR2N7485U3 | International Rectifier | VDS : 130V. VGS : 20V. IDS : 12.5A. Power:75W. Tjmax : 150°C. Radiation: 100kRad(Si). | 0.80. 0.75. 0.75. 0.65. 110°C. 1. | 104 V. 15 V. 9.375A. 48.75 W. 110°C. 1. | MOSFET for flyback DC/DC converter |

Table 16 Bipolar NPN Transistor Group Code 12-01

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| 2N4957 JANS2N4957UB | Semicoa | VCEO : 30V. VCBO : 30V. VEBO : 3V. IC : 30mA. Power:200mW. Tjmax : 200°C. Radiation:. 100kRad(Si). | 0.75. 0.75. 0.75. 0.75. 0.65. 110°C. 1. | 22.5 V. 22.5 V. 2.25 V. 22.5mA. 130mW. 110°C. 1. | PNP RF-transistor for preamp |

Table 17 UHF NPN Transistor Group Code 12-01

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| 2N2857 JANS2N2857UB | Semicoa | VCEO : 15V. VCBO : 30V. VEBO : 3V. IC : 40mA. Power:300m W. Tjmax : 200°C. Radiation:. 100kRad(Si). | 0.75. 0.75. 0.75. 0.75. 0.65. 110°C. 1. | 11.25 V. 22.5 V. 2.25 V. 30mA. 195mW. 110°C. 1. | NPN RF-transistor for preamp |

Table 18 Bipolar NPN Transistor Group Code 12-01

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| 2N5551 SOC5551SW LCC-3 | STMicroelectronics | VCEO : 160V. VCBO : 180V. VEBO : 6 V. IC : 0.5A. Power:0.36 W. T jmax : 200°C. Radiation: 100kRad(Si). | 0.75. 0.75. 0.75. 0.75. 0.65. 110°C. 1. | 120 V. 135 V. 4.5 V. 0.375 A. 0.234 W. 110°C. | 160V NPN transistor for HET-EPT BIAS current source to prevent damage of the JFETs in preamplifier. |

Table 19 Linear Operational Amplifier Group Code 08-50

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| RH1498MW | Linear Technology | VCC : 36V. VIN : 15V. IOUT : 1.8mA. T jmax : 125°C. | 0.9. 0.70. 0.8. 110°C. | 32.4 V. 10.5 V. 1.44mA. 110°C. | Standard Op-Amp |

Table 20 Linear Operational Amplifier Group Code 08-50

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| RH1056AMW | Linear Technology | VCC : 20V. VIN : 20V. IOUT : 6.2mA. T jmax : 150°C. | 0.9. 0.7. 0.8. 110°C. | 18 V. 14 V. 4.96mA. 110°C. | Bias leakage current housekeeping Op-Amp |

Table 21 Memory SRAM Group Code 08-20

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| 5962R0422701VXC | Aeroflex | VCC : 3,8V. IOUT : ~. T jmax : 150°C. | 0.9. 0.8. 110°C. | 3.42V. ~. 110°C. | FPGAs ext. SRAM |

Table 22 Memory EEPROM Group Code 08-24

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| 28LV010RPFS25 | Maxwell | VCC : 7V. IOUT : ~. T jmax : 125°C. | 0.9. 0.8. 110°C. | 6.3V. ~. 110°C. | EEPROM for configuration data |

Table 23 Switching Regulator Group Code 08-54

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| MSK5059RHG | Maxwell | Vin : 16V. IOUT : 4.5A. T jmax : 150°C. Radiation: 100kRad(Si. | 0.9. 0.8. 110°C. 1. | 14.4 V. 3.6 A. 110°C. 1. | switching regulator FPGA-CORE voltage 1.5V, SRAM, ADC, FPGA-IO 3.3V |

Table 24 PWM Controller Group Code 08-54

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| ISL78841ASEHQF | Maxwell | VCC : 30V. IOUT : 1A. T jmax : 150°C. Radiation: 100kRad(Si. | 0.9. 0.8. 110°C. 1. | 27V. 0.8A. 110°C. 1. | LVPS DC/DC converter |

Table 25 Counter/Divider Group Code 08-54

| Generic Part Type | Manufacturer | Rated Parameters | Derating Factor | Derated Value | Application |
| --- | --- | --- | --- | --- | --- |
| HCC4024BK | Maxwell | VCC : 22V. VIN : 22.5V. Power Dis :200mW. T jmax : 150°C. Radiation: 100kRad(Si). | 0.9. 0.9. 0.8. 110°C. 1. | 19.8V. 20.25 V. 0.16W. 110°C. 1. | clock divider for DC/DC converter clock from 1.6MHz crystal oscillator to 100kHz |